IoT-Based Smart Doorbell System

Using ESP32-CAM with Telegram Integration

| Institute | Institute of Engineering and Technology, DAVV Indore |
| --- | --- |
| Subject | Internet of Things / Electronics & Telecommunication |
| Project Title | IoT-Based Smart Doorbell System using ESP32-CAM |
| Team Members | Pushti Bagla — 23T6149 |
|  | Suhani Lahoti — 23T6165 |
|  | Utkarsh Tayade — 23T6171 |
|  | Yash Bhoje — 23T6175 |

Table of Contents

1. Project Overview 3 2. System Components 4 3. System Architecture & GPIO Pin Mapping 5 4. Core Functionality & Working 6 5. Source Code — Part 1: Libraries, Definitions & Setup 7 6. Source Code — Part 2: Functions (OLED, Camera, Web, Telegram) 8 7. Source Code — Part 3: Setup() & Loop() — Main Execution 10 8. Telegram Bot Interface & Commands 11 9. Conclusion & Project Summary 12 10. References 13

1. Project Overview

This IoT-based system transforms a traditional doorbell into a smart security solution using the ESP32-CAM microcontroller module. When a visitor presses the doorbell button, the ESP32-CAM captures an image and instantly sends an alert via Telegram over a WiFi connection. The homeowner can then remotely grant or deny access through simple Telegram commands. A locally hosted web server provides a live camera feed accessible from any browser on the same network.

1.1  Problem Statement

Traditional doorbells only alert the person physically present at home. Modern lifestyles demand remote monitoring and access control. This project addresses the need for a low-cost, internet-connected smart doorbell that notifies the owner anywhere in the world and allows them to control door access remotely without any dedicated cloud subscription.

1.2  Key Features

- Real-time visitor monitoring via image capture on button press

- Remote access control through Telegram commands (/open, /deny)

- Automatic image capture with flash illumination for quality photos

- Live video streaming accessible from any browser on the local network

- OLED status display showing boot, WiFi, and door state information

- Cost-effective implementation — total hardware cost under ₹800

- Scalable design suitable for homes, offices, and small businesses

1.3  Technologies & Tools Used

| Hardware Platform | ESP32-CAM (AI Thinker) with OV2640 camera module |
| --- | --- |
| Firmware Language | Arduino C++ (compiled via Arduino IDE) |
| Communication | WiFi 802.11 b/g/n, HTTPS via WiFiClientSecure |
| Messaging Platform | Telegram Bot API (UniversalTelegramBot library) |
| Display | SSD1306 OLED 128×64 via I2C (Adafruit library) |
| Web Interface | ESP32 WebServer library hosting live feed on port 80 |
| Image Processing | img_converters (frame2bmp) for BMP conversion |
| Power Supply | 5V USB or external adapter |

2. System Components

The system is built using readily available, low-cost components. The table below lists each component, its purpose, and the specific electrical connections used in the circuit.

| Component | Description | Connection / Pin Details |
| --- | --- | --- |
| ESP32-CAM Module | Core controller with OV2640 camera and built-in WiFi (2.4 GHz). | GPIO 0/2/4/12/13 (I/O), I2C (GPIO 14/15), 5V power, UART for programming |
| FTDI Board (USB-to-Serial) | USB-to-Serial converter for programming the ESP32-CAM via Arduino IDE. | TX→RX, RX→TX, GND→GND, VCC→3.3V |
| OLED Display (SSD1306) | 128×64 I2C OLED showing boot status, WiFi connection, and door state messages. | SDA→GPIO 14, SCL→GPIO 15, VCC→3.3V, GND |
| Push Button (Doorbell) | Momentary tactile switch; triggers image capture and Telegram notification on press. | One pin→GPIO 2 (INPUT_PULLUP), other pin→GND |
| Green LED + 220Ω Resistor | Indicates door OPEN state (access granted). | Anode→GPIO 12 via 220Ω, Cathode→GND |
| Red LED + 220Ω Resistor | Indicates door CLOSED or ACCESS DENIED state. | Anode→GPIO 13 via 220Ω, Cathode→GND |
| Built-in Flash LED (GPIO 4) | Illuminates scene during image capture for better photo quality. | Controlled via GPIO 4 (on-board) |
| 5V Power Supply | USB or external 5V adapter powering the entire system. | Connected to ESP32-CAM 5V pin |

2.1  Notes on Component Selection

- The ESP32-CAM (AI Thinker variant) was chosen for its integrated OV2640 2MP camera, built-in flash LED on GPIO 4, and dual-core 240 MHz Xtensa processor — all in a single compact module.

- SSD1306 OLED was placed on I2C pins GPIO 14 (SDA) and GPIO 15 (SCL) because the standard I2C pins (GPIO 21/22) conflict with camera data lines.

- 220Ω current-limiting resistors protect both LEDs from overcurrent at 3.3V GPIO output.

- The FTDI board is only needed during programming; it is disconnected during normal operation.

3. System Architecture & GPIO Pin Mapping

3.1  Block Diagram (Text Representation)

The system follows a simple peripheral-to-ESP32-CAM architecture. The central controller manages all peripherals via GPIO pins while maintaining continuous WiFi connectivity for Telegram polling and web server hosting.

[Push Button GPIO 2] ─────────────────────────────> INPUT to ESP32-CAM [SSD1306 OLED] ─── SDA:GPIO14 / SCL:GPIO15 ──────> I2C Display [Green LED] ───── GPIO 12 via 220Ω ───────────────> Door Open Indicator [Red LED] ─────── GPIO 13 via 220Ω ───────────────> Door Closed / Denied [Flash LED] ───── GPIO 4 (on-board) ──────────────> Capture illumination [OV2640 Camera] ─ Parallel interface (GPIO 5-36) ─> Image capture [5V Power Supply] ──────────────────────────────── Powers all modules ESP32-CAM (AI Thinker) ──[WiFi]──> Telegram Bot API ──[WiFi]──> Web Browser (Live Feed http://IP/)

3.2  GPIO Pin Mapping Table

| GPIO Pin | Connected To | Mode | Notes |
| --- | --- | --- | --- |
| GPIO 2 | Push Button | INPUT_PULLUP | Active LOW — triggers doorbell on press |
| GPIO 4 | Flash LED (built-in) | OUTPUT | HIGH before capture, LOW after |
| GPIO 12 | Green LED | OUTPUT | HIGH = Door Open, LOW = Closed |
| GPIO 13 | Red LED | OUTPUT | HIGH = Door Closed/Denied, LOW = Open |
| GPIO 14 | OLED SDA (I2C) | I2C Data | Wire.begin(14, 15) — SSD1306 |
| GPIO 15 | OLED SCL (I2C) | I2C Clock | Wire.begin(14, 15) — SSD1306 |
| GPIO 0 | Camera XCLK | Camera internal | Part of OV2640 camera interface |
| GPIO 26/27 | Camera SIOD/SIOC | Camera I2C | OV2640 SCCB configuration lines |
| GPIO 25 | Camera VSYNC | Camera sync | Vertical sync for frame timing |
| GPIO 23 | Camera HREF | Camera sync | Horizontal reference signal |
| GPIO 22 | Camera PCLK | Camera clock | Pixel clock output |

4. Core Functionality & Working

4.1  System Boot Sequence

- Serial monitor initialised at 115200 baud.

- GPIO pins configured: LEDs as OUTPUT, Button as INPUT_PULLUP, Flash as OUTPUT.

- OLED initialised via I2C; displays 'Booting...' message.

- Camera configured with QQVGA frame size and RGB565 pixel format and initialised.

- WiFi connection established; OLED displays 'Connecting WiFi' then 'WiFi Connected'.

- Telegram bot sends 'System Ready' message to configured CHAT_ID.

- Web server routes registered ('/capture', '/') and server started on port 80.

4.2  Visitor Detection Flow

- Button is connected to GPIO 2 with INPUT_PULLUP (reads HIGH when idle).

- handleButtonPress() runs every loop iteration, tracking lastState vs currentState.

- On HIGH→LOW transition (button pressed), sendPhotoTelegram() is called.

- A 300ms debounce delay prevents multiple triggers from a single press.

4.3  Image Capture and Transmission

- Flash LED (GPIO 4) turns HIGH for 150ms to illuminate the visitor.

- The local capture URL (http://[IP]/capture) is sent to the Telegram CHAT_ID.

- On /capture web request, esp_camera_fb_get() retrieves frame buffer from OV2640.

- frame2bmp() converts the RGB565 frame to BMP format in memory.

- BMP data is sent as HTTP response (image/bmp Content-Type) to the browser.

- Frame buffer is released with esp_camera_fb_return() to free memory.

4.4  Telegram Polling Loop

- handleTelegram() is called every loop iteration.

- bot.getUpdates() polls Telegram servers for new messages via HTTPS.

- Incoming message text is parsed and matched against /capture, /open, /deny commands.

- Responses are sent back via bot.sendMessage() with status confirmations.

4.5  Live Web Feed

- ESP32-CAM hosts a web server on port 80 accessible via http://[IP_ADDRESS]/.

- The root route (/) serves an HTML page with an auto-refreshing image tag.

- The /capture endpoint serves a fresh BMP image on every HTTP GET request.

- Any device on the same WiFi network can view the live feed from a browser.

5. Source Code — Part 1: Libraries, Definitions & Global Setup

This section includes all library inclusions, macro definitions for GPIO pins and Telegram credentials, and global object declarations. These definitions set up the entire hardware and software environment used throughout the program.

{ } Part 1 — Libraries, Pin Definitions & Global Objects

#include "esp_camera.h" // ESP32-CAM camera driver

#include <WiFi.h> // WiFi connectivity

#include <WebServer.h> // Local HTTP server on port 80

#include "img_converters.h" // frame2bmp() for image conversion

#include <WiFiClientSecure.h> // Secure HTTPS client for Telegram

#include <UniversalTelegramBot.h>// Telegram Bot API wrapper

// ─── OLED Libraries ───────────────────────────────────────────

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

// ─── WiFi Credentials ─────────────────────────────────────────

const char* ssid = "YOUR_WIFI_SSID";

const char* password = "YOUR_WIFI_PASSWORD";

// ─── Telegram Bot Credentials ─────────────────────────────────

#define BOTtoken "YOUR_BOT_TOKEN_HERE"

#define CHAT_ID "YOUR_CHAT_ID_HERE"

WiFiClientSecure client;

UniversalTelegramBot bot(BOTtoken, client);

// ─── LED Pin Definitions ───────────────────────────────────────

#define GREEN_LED 12

#define RED_LED 13

// ─── Button & Flash ────────────────────────────────────────────

#define BUTTON_PIN 2

#define FLASH_LED 4

// ─── OLED Display ──────────────────────────────────────────────

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

// ─── Camera GPIO Pin Map (AI Thinker ESP32-CAM) ────────────────

#define PWDN_GPIO_NUM 32

#define RESET_GPIO_NUM -1

#define XCLK_GPIO_NUM 0

#define SIOD_GPIO_NUM 26

#define SIOC_GPIO_NUM 27

#define Y9_GPIO_NUM 35 #define Y8_GPIO_NUM 34

#define Y7_GPIO_NUM 39 #define Y6_GPIO_NUM 36

#define Y5_GPIO_NUM 21 #define Y4_GPIO_NUM 19

#define Y3_GPIO_NUM 18 #define Y2_GPIO_NUM 5

#define VSYNC_GPIO_NUM 25

#define HREF_GPIO_NUM 23

#define PCLK_GPIO_NUM 22

WebServer server(80); // HTTP server on port 80

6. Source Code — Part 2: Functions

This section contains all the core function implementations: OLED helper, image capture handler, web page handler, Telegram photo sender, Telegram command processor, serial trigger handler, and button press handler.

{ } Part 2a — OLED Helper Function

void oledPrint(String msg) {

display.clearDisplay();

display.setCursor(0, 0);

display.println(msg);

display.display();

}

{ } Part 2b — Image Capture Web Handler (/capture endpoint)

void handle_capture() {

camera_fb_t* fb = esp_camera_fb_get(); // Grab frame from camera

if (!fb) {

server.send(500, "text/plain", "Camera capture failed");

return;

}

uint8_t* bmp_buf = NULL;

size_t bmp_len = 0;

if (!frame2bmp(fb, &bmp_buf, &bmp_len)) { // Convert to BMP

server.send(500, "text/plain", "BMP conversion failed");

esp_camera_fb_return(fb);

return;

}

server.sendHeader("Content-Type", "image/bmp");

server.send_P(200, "image/bmp", (const char*)bmp_buf, bmp_len);

free(bmp_buf);

esp_camera_fb_return(fb); // Release frame buffer

}

{ } Part 2c — Web Root Handler (Live Feed Page)

void handle_root() {

String html = "<html><head><title>ESP32-CAM Live</title></head>";

html += "<body><h2>ESP32-CAM Live Feed</h2>";

html += "<img src='/capture' width='320' height='240'/>";

html += "</body></html>";

server.send(200, "text/html", html);

}

{ } Part 2d — Send Photo to Telegram

void sendPhotoTelegram() {

digitalWrite(FLASH_LED, HIGH); // Flash ON for 150ms

delay(150);

String url = "http://" + WiFi.localIP().toString() + "/capture";

bot.sendMessage(CHAT_ID, "Visitor:\n" + url, ""); // Send URL to Telegram

digitalWrite(FLASH_LED, LOW); // Flash OFF

oledPrint("Doorbell Pressed");

}

{ } Part 2e — Telegram Command Handler

void handleTelegram() {

int numNewMessages = bot.getUpdates(bot.last_message_received + 1);

while (numNewMessages) {

for (int i = 0; i < numNewMessages; i++) {

String text = bot.messages[i].text;

if (text == "/capture") {

sendPhotoTelegram();

}

if (text == "/open") {

digitalWrite(GREEN_LED, HIGH);

digitalWrite(RED_LED, LOW);

bot.sendMessage(CHAT_ID, "Door Opened (10 sec)", "");

oledPrint("Door Opened");

delay(10000); // Hold open for 10 seconds

digitalWrite(GREEN_LED, LOW);

digitalWrite(RED_LED, HIGH);

bot.sendMessage(CHAT_ID, "Door Closed", "");

oledPrint("Door Closed");

}

if (text == "/deny") {

digitalWrite(GREEN_LED, LOW);

digitalWrite(RED_LED, HIGH);

bot.sendMessage(CHAT_ID, "Access Denied", "");

oledPrint("Access Denied");

}

}

numNewMessages = bot.getUpdates(bot.last_message_received + 1);

}

}

{ } Part 2f — Serial Trigger & Button Press Handlers

// ─── Serial Trigger (type 'c' in Serial Monitor) ──────────────

void handleSerialTrigger() {

if (Serial.available()) {

char input = Serial.read();

if (input == 'c') {

Serial.println("Capture triggered");

sendPhotoTelegram();

}

}

}

// ─── Button Debounce & Press Detection ────────────────────────

void handleButtonPress() {

static bool lastState = HIGH;

bool currentState = digitalRead(BUTTON_PIN);

if (lastState == HIGH && currentState == LOW) { // Falling edge

Serial.println("Doorbell Pressed");

sendPhotoTelegram();

delay(300); // 300ms debounce

}

lastState = currentState;

}

7. Source Code — Part 3: Setup() & Loop()

The setup() function runs once on power-on and initialises every peripheral in sequence. The loop() function runs continuously and handles all four tasks: web server clients, Telegram polling, serial trigger, and button monitoring.

{ } Part 3 — void setup() and void loop()

void setup() {

Serial.begin(115200);

// ─── GPIO Initialisation ──────────────────────────────────

pinMode(GREEN_LED, OUTPUT); digitalWrite(GREEN_LED, LOW);

pinMode(RED_LED, OUTPUT); digitalWrite(RED_LED, HIGH); // Default: door closed

pinMode(BUTTON_PIN, INPUT_PULLUP);

pinMode(FLASH_LED, OUTPUT); digitalWrite(FLASH_LED, LOW);

// ─── OLED Initialisation ──────────────────────────────────

Wire.begin(14, 15); // SDA=14, SCL=15 (non-default I2C pins)

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println("OLED init failed"); }

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(WHITE);

oledPrint("Booting...");

// ─── Camera Configuration ─────────────────────────────────

camera_config_t config;

config.ledc_channel = LEDC_CHANNEL_0;

config.ledc_timer = LEDC_TIMER_0;

config.pin_d0 = Y2_GPIO_NUM; config.pin_d1 = Y3_GPIO_NUM;

config.pin_d2 = Y4_GPIO_NUM; config.pin_d3 = Y5_GPIO_NUM;

config.pin_d4 = Y6_GPIO_NUM; config.pin_d5 = Y7_GPIO_NUM;

config.pin_d6 = Y8_GPIO_NUM; config.pin_d7 = Y9_GPIO_NUM;

config.pin_xclk = XCLK_GPIO_NUM;

config.pin_pclk = PCLK_GPIO_NUM;

config.pin_vsync = VSYNC_GPIO_NUM;

config.pin_href = HREF_GPIO_NUM;

config.pin_sccb_sda = SIOD_GPIO_NUM;

config.pin_sccb_scl = SIOC_GPIO_NUM;

config.pin_pwdn = PWDN_GPIO_NUM;

config.pin_reset = RESET_GPIO_NUM;

config.xclk_freq_hz = 20000000;

config.pixel_format = PIXFORMAT_RGB565;

config.frame_size = FRAMESIZE_QQVGA; // 160x120 resolution

config.fb_count = 1;

if (esp_camera_init(&config) != ESP_OK) {

Serial.println("Camera init failed");

oledPrint("Camera Failed"); return; }

oledPrint("Camera Ready");

// ─── WiFi Connection ──────────────────────────────────────

WiFi.begin(ssid, password);

oledPrint("Connecting WiFi");

while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); }

Serial.println(WiFi.localIP());

oledPrint("WiFi Connected");

// ─── Telegram Initialisation ──────────────────────────────

client.setInsecure(); // Skip SSL certificate verification

bot.sendMessage(CHAT_ID, "System Ready", "");

oledPrint("System Ready");

// ─── Web Server Route Registration ────────────────────────

server.on("/", handle_root);

server.on("/capture", handle_capture);

server.begin();

Serial.println("Server started — type 'c' to capture");

}

// ─── Main Loop ────────────────────────────────────────────────

void loop() {

server.handleClient(); // Serve web requests

handleTelegram(); // Poll Telegram for commands

handleSerialTrigger(); // Check Serial Monitor input

handleButtonPress(); // Detect doorbell button press

}

8. Telegram Bot Interface & Commands

The system uses the UniversalTelegramBot library to interact with the Telegram API over HTTPS. The bot polls for new messages in every loop iteration. Three commands are supported, each triggering hardware output actions and sending confirmation messages back to the user.

8.1  Setting Up the Telegram Bot

- Open Telegram and search for @BotFather.

- Send /newbot, enter a bot name and username to receive the BOT TOKEN.

- Search for @userinfobot and send /start to get your personal CHAT_ID.

- Replace the BOTtoken and CHAT_ID values in the code with your credentials.

8.2  Available Commands

| Command | Action | Hardware Behaviour |
| --- | --- | --- |
| /capture | Triggers image capture immediately. Flash LED turns ON for 150ms, photo is captured, and the image URL is sent to CHAT_ID. | Flash ON → capture → send URL → Flash OFF |
| /open | Grants access. Green LED turns HIGH (door open), Red LED turns LOW. After 10 seconds, Green LED turns LOW, Red LED turns HIGH automatically. | GREEN=HIGH → 10s delay → GREEN=LOW, RED=HIGH |
| /deny | Denies access. Green LED stays LOW, Red LED stays HIGH. Access Denied message sent back to user via Telegram. | GREEN=LOW, RED=HIGH, sendMessage(Access Denied) |

8.3  Live Feed Access

Access the live camera feed by opening a web browser on any device connected to the same WiFi network and visiting: http://[ESP32_IP_ADDRESS]/ The IP address is printed to the Serial Monitor after WiFi connects, and also displayed briefly on the OLED. The feed auto-refreshes by serving a fresh BMP capture on each /capture HTTP GET request.

9. Conclusion & Project Summary

This IoT-based Smart Doorbell System successfully demonstrates how a microcontroller with integrated WiFi and camera can be used to build a practical, remote-access security solution at minimal cost. The project integrates hardware peripherals, embedded programming, cloud messaging, and web server hosting into a single cohesive system.

9.1  Results Achieved

- Successfully captured and transmitted visitor images to Telegram on doorbell press.

- Remote access grant and denial working reliably with LED status feedback.

- OLED display correctly reflects all system states throughout operation.

- Web server streaming live camera feed accessible from any browser on local network.

- System boots and becomes operational within approximately 8–12 seconds from power-on.

9.2  Advantages

- No recurring subscription cost — uses free Telegram API.

- Works on any existing WiFi network without additional infrastructure.

- Total hardware cost is significantly lower than commercial smart doorbells.

- Code is modular and easily extensible (motion detection, multi-user support, SD logging).

9.3  Limitations & Future Scope

- Currently requires LAN access for live video feed; public URL needs port forwarding or DDNS.

- Telegram polling introduces ~1–2 second command response latency.

- Image quality limited to QQVGA (160×120) due to memory constraints; can be improved with PSRAM-enabled board.

- Future enhancement: Face recognition using the ESP32 face detection library for automatic known-visitor identification.

- Future enhancement: SD card logging of all captured images with timestamps.

- Future enhancement: Multiple CHAT_ID support for family-wide notifications.

10. References

| [1] | Espressif Systems — ESP32-CAM Datasheet and Technical Reference Manual. Available at: https://docs.espressif.com |
| --- | --- |
| [2] | AI Thinker — ESP32-CAM Module Pinout and Schematic. Available at: https://github.com/raphaelbs/esp32-cam-ai-thinker |
| [3] | Randomnerdtutorials.com — ESP32-CAM Take Photo and Send to Telegram. Accessed April 2025. |
| [4] | Adafruit Industries — Adafruit SSD1306 Library Documentation. Available at: https://learn.adafruit.com/monochrome-oled-breakouts |
| [5] | UniversalTelegramBot Library — Brian Lough. Available at: https://github.com/witnessmenow/Universal-Arduino-Telegram-Bot |
| [6] | Arduino IDE — Arduino Open-Source Platform. Available at: https://www.arduino.cc/en/software |
| [7] | Telegram BotFather Documentation — Creating Telegram Bots. Available at: https://core.telegram.org/bots |
| [8] | ESP32 Camera Web Server Example — Espressif GitHub Repository. Available at: https://github.com/espressif/arduino-esp32/tree/master/libraries/ESP32/examples/Camera |